DEPARTMENT OF ROBOTICS & ARTIFICIAL INTELLIGENCE

Total Marks:

Obtained Marks:

# Artificial Neural Networks Lab

## Project

### Submitted To:	Mr. Hassan Mujtaba Student Name:		Aqsa Ehsan

### Registration Number: 23108336

DEPARTMENT OF ROBOTICS & ARTIFICIAL INTELLIGENCE

### Topic : Audio Tampering Detection and Localization using AI Techniques

### Code

### # ============================================================

### # INSTALL REQUIRED LIBRARIES

### # ============================================================

### !pip install -q numpy pandas matplotlib scipy scikit-learn

### # ============================================================

### # IMPORTS

### # ============================================================

### import os

### import csv

### import time

### import random

### import warnings

### warnings.filterwarnings("ignore")

### import numpy as np

### import pandas as pd

### import matplotlib

### matplotlib.use("Agg")

### import matplotlib.pyplot as plt

### import matplotlib.patches as mpatches

### import scipy.signal as ssig

### from scipy.fftpack import dct

### from sklearn.model_selection import cross_val_score, StratifiedKFold

### from sklearn.preprocessing import StandardScaler

### from sklearn.decomposition import PCA

### from sklearn.pipeline import Pipeline

### from sklearn.neural_network import MLPClassifier

### from sklearn.svm import SVC

### from sklearn.ensemble import RandomForestClassifier, GradientBoostingClassifier

### from sklearn.metrics import (

### accuracy_score,

### precision_score,

### recall_score,

### f1_score,

### classification_report,

### confusion_matrix,

### )

### # ============================================================

### # RANDOM SEED

### # ============================================================

### random.seed(42)

### np.random.seed(42)

### # ============================================================

### # GLOBAL SETTINGS

### # ============================================================

### SR = 22050

### DURATION = 3

### N_SAMPLES = SR * DURATION

### N_MFCC = 40

### N_FFT = 2048

### HOP_LEN = 512

### N_MELS = 64

### OUTPUT_DIR = "outputs"

### os.makedirs(OUTPUT_DIR, exist_ok=True)

### DATASET_CSV = "dataset.csv"

### RESULTS_CSV = "results.csv"

### # ============================================================

### # SYNTHETIC DATA GENERATOR

### # ============================================================

### class SyntheticDataGenerator:

### TAMPERING_TYPES = [

### "splice",

### "cut",

### "noise_burst",

### "compression"

### ]

### def __init__(self, n_total=200):

### self.n_total = n_total

### self.n_clean = n_total // 2

### self.n_tampered = n_total - self.n_clean

### def _base_signal(self):

### t = np.linspace(0, DURATION, N_SAMPLES, endpoint=False)

### f0 = np.random.uniform(80, 300)

### sig = np.zeros(N_SAMPLES)

### for k in range(1, 7):

### amp = 1.0 / k

### sig += amp * np.sin(

### 2 * np.pi * f0 * k * t +

### np.random.uniform(0, np.pi)

### )

### sig += np.random.randn(N_SAMPLES) * 0.02

### sig = sig / (np.max(np.abs(sig)) + 1e-9)

### return sig.astype(np.float32)

### def _apply_splice(self, sig):

### out = sig.copy()

### start = np.random.randint(SR // 2, N_SAMPLES - SR)

### length = np.random.randint(SR // 4, SR)

### replacement = self._base_signal()[start:start + length]

### out[start:start + length] = replacement

### return out

### def _apply_cut(self, sig):

### start = np.random.randint(SR // 4, N_SAMPLES // 2)

### length = np.random.randint(SR // 4, SR)

### out = np.concatenate([

### sig[:start],

### sig[start + length:]

### ])

### out = np.pad(out, (0, N_SAMPLES - len(out)))

### return out.astype(np.float32)

### def _apply_noise_burst(self, sig):

### out = sig.copy()

### start = np.random.randint(0, N_SAMPLES - SR // 2)

### length = np.random.randint(SR // 8, SR // 2)

### out[start:start + length] += np.random.randn(length) * 0.5

### out = out / (np.max(np.abs(out)) + 1e-9)

### return out.astype(np.float32)

### def _apply_compression(self, sig):

### bits = np.random.choice([4, 6, 8])

### steps = 2 ** bits

### out = np.round(sig * steps) / steps

### return out.astype(np.float32)

### def generate(self):

### signals = {}

### rows = []

### # CLEAN

### for i in range(self.n_clean):

### sid = f"clean_{i:04d}"

### signals[sid] = self._base_signal()

### rows.append({

### "id": sid,

### "label": "clean",

### "tampering_type": "none"

### })

### # TAMPERED

### for i in range(self.n_tampered):

### sid = f"tampered_{i:04d}"

### ttype = self.TAMPERING_TYPES[i % 4]

### base = self._base_signal()

### if ttype == "splice":

### tampered = self._apply_splice(base)

### elif ttype == "cut":

### tampered = self._apply_cut(base)

### elif ttype == "noise_burst":

### tampered = self._apply_noise_burst(base)

### else:

### tampered = self._apply_compression(base)

### signals[sid] = tampered

### rows.append({

### "id": sid,

### "label": "tampered",

### "tampering_type": ttype

### })

### random.shuffle(rows)

### split_idx = int(0.8 * len(rows))

### for i, row in enumerate(rows):

### row["split"] = "train" if i < split_idx else "test"

### df = pd.DataFrame(rows)

### df.to_csv(DATASET_CSV, index=False)

### print("Dataset generated successfully")

### return signals, df

### # ============================================================

### # PREPROCESSING

### # ============================================================

### class PreprocessingModule:

### def normalize(self, sig):

### return sig / (np.max(np.abs(sig)) + 1e-9)

### def process(self, sig):

### sig = self.normalize(sig)

### if len(sig) < N_SAMPLES:

### sig = np.pad(sig, (0, N_SAMPLES - len(sig)))

### else:

### sig = sig[:N_SAMPLES]

### return sig.astype(np.float32)

### # ============================================================

### # FEATURE EXTRACTION

### # ============================================================

### class FeatureExtractionModule:

### def __init__(self):

### self._build_filterbank()

### def _build_filterbank(self):

### def hz2mel(h):

### return 2595 * np.log10(1 + h / 700)

### def mel2hz(m):

### return 700 * (10 ** (m / 2595) - 1)

### low = hz2mel(80)

### high = hz2mel(SR / 2)

### mel_points = np.linspace(low, high, N_MELS + 2)

### hz_points = mel2hz(mel_points)

### bins = np.floor((N_FFT + 1) * hz_points / SR).astype(int)

### self.fb = np.zeros((N_MELS, N_FFT // 2 + 1))

### for m in range(1, N_MELS + 1):

### f1 = bins[m - 1]

### f2 = bins[m]

### f3 = bins[m + 1]

### for k in range(f1, f2):

### self.fb[m - 1, k] = (k - f1) / (f2 - f1 + 1e-9)

### for k in range(f2, f3):

### self.fb[m - 1, k] = (f3 - k) / (f3 - f2 + 1e-9)

### def _stft(self, sig):

### frames = []

### for i in range(0, len(sig) - N_FFT, HOP_LEN):

### frame = sig[i:i + N_FFT]

### frame = frame * np.hanning(N_FFT)

### spec = np.abs(np.fft.rfft(frame))

### frames.append(spec)

### return np.array(frames).T

### def extract(self, sig):

### mag = self._stft(sig)

### mel = self.fb @ mag

### log_mel = np.log(mel + 1e-9)

### mfcc = dct(log_mel, axis=0, norm="ortho")[:N_MFCC]

### feat = np.concatenate([

### mfcc.mean(axis=1),

### mfcc.std(axis=1)

### ])

### return feat.astype(np.float32)

### def extract_batch(self, signals, df):

### X = []

### y = []

### ids = []

### for _, row in df.iterrows():

### vec = self.extract(signals[row["id"]])

### X.append(vec)

### y.append(1 if row["label"] == "tampered" else 0)

### ids.append(row["id"])

### return np.array(X), np.array(y), ids

### # ============================================================

### # TRAINING

### # ============================================================

### class TrainingModule:

### MODELS = {

### "ANN": MLPClassifier(

### hidden_layer_sizes=(256, 128),

### max_iter=200,

### early_stopping=True,

### random_state=42

### ),

### "SVM": SVC(

### probability=True,

### random_state=42

### ),

### "RF": RandomForestClassifier(

### n_estimators=100,

### random_state=42

### ),

### "GBM": GradientBoostingClassifier(

### random_state=42

### )

### }

### def __init__(self):

### self.pipelines = {}

### def build(self, n_features):

### pca_components = min(40, n_features)

### for name, clf in self.MODELS.items():

### self.pipelines[name] = Pipeline([

### ("scaler", StandardScaler()),

### ("pca", PCA(

### n_components=pca_components,

### random_state=42

### )),

### ("clf", clf)

### ])

### def train(self, X_train, y_train):

### best_acc = 0

### best_model = None

### best_name = None

### for name, pipe in self.pipelines.items():

### print(f"\nTraining {name}")

### scores = cross_val_score(

### pipe,

### X_train,

### y_train,

### cv=5,

### scoring="accuracy"

### )

### print("Accuracy:", scores.mean())

### pipe.fit(X_train, y_train)

### if scores.mean() > best_acc:

### best_acc = scores.mean()

### best_model = pipe

### best_name = name

### print("\nBest Model:", best_name)

### return best_model, best_name

### # ============================================================

### # EVALUATION

### # ============================================================

### class EvaluationModule:

### def evaluate(self, model, X_test, y_test):

### pred = model.predict(X_test)

### probs = model.predict_proba(X_test)

### acc = accuracy_score(y_test, pred)

### print("\nAccuracy:", acc)

### print("\nClassification Report:\n")

### print(classification_report(

### y_test,

### pred,

### target_names=["clean", "tampered"]

### ))

### cm = confusion_matrix(y_test, pred)

### plt.figure(figsize=(5, 4))

### plt.imshow(cm, cmap="Blues")

### plt.title("Confusion Matrix")

### plt.colorbar()

### plt.xticks([0,1], ["clean", "tampered"])

### plt.yticks([0,1], ["clean", "tampered"])

### for i in range(2):

### for j in range(2):

### plt.text(j, i, cm[i,j],

### ha="center",

### va="center",

### color="black")

### plt.savefig(f"{OUTPUT_DIR}/confusion_matrix.png")

### plt.close()

### print("Confusion matrix saved")

### # ============================================================

### # MAIN

### # ============================================================

### def main():

### print("\nAUDIO TAMPERING DETECTION SYSTEM\n")

### # DATASET

### generator = SyntheticDataGenerator()

### signals, df = generator.generate()

### # PREPROCESSING

### pre = PreprocessingModule()

### processed = {}

### for sid, sig in signals.items():

### processed[sid] = pre.process(sig)

### # FEATURES

### fe = FeatureExtractionModule()

### X, y, ids = fe.extract_batch(processed, df)

### print("Feature Shape:", X.shape)

### # SPLIT

### train_ids = set(df[df["split"] == "train"]["id"])

### train_mask = np.array([i in train_ids for i in ids])

### test_mask = ~train_mask

### X_train = X[train_mask]

### y_train = y[train_mask]

### X_test = X[test_mask]

### y_test = y[test_mask]

### # TRAINING

### trainer = TrainingModule()

### trainer.build(X.shape[1])

### best_model, best_name = trainer.train(

### X_train,

### y_train

### )

### # EVALUATION

### evaluator = EvaluationModule()

### evaluator.evaluate(

### best_model,

### X_test,

### y_test

### )

### print("\nDONE SUCCESSFULLY")

### # ============================================================

### # RUN

### # ============================================================

### if __name__ == "__main__":

### main()

#### THE END

====================================